हो—या SIH26126 साठी जिंकण्यासारखी solution direction साध्या “camera + YOLO + SLAM + obstacle avoidance” पेक्षा खूप पुढे नेऊ शकतो. तुमची core innovation अशी असावी: UGV ने फक्त वस्तू ओळखू नयेत; तर स्वतःच्या प्रत्यक्ष terrain-interaction मधून कोणता मार्ग खरोखर सुरक्षित, चालण्याजोगा आणि वाहनासाठी योग्य आहे हे शिकावे—आणि uncertainty वाढली की सुरक्षितपणे थांबावे किंवा human-in-the-loop कडे मदत मागावी.

समस्या काय आहे?

BEL च्या SIH26126 मध्ये GPS-denied outdoor environment मध्ये camera feed हे मुख्य sensor ठेवून UGV ला Point A ते Point B पर्यंत collision-free नेणे अपेक्षित आहे. अधिकृत statement मध्ये तीन स्पष्ट अपेक्षा आहेत: real-time safe-path/hazard detection, GPS शिवाय visual localization, आणि अचानक आलेल्या obstacle भोवती dynamic rerouting; deliverable म्हणून lightweight perception AI, visual SLAM/odometry, आणि wheel/motor commands देणारा planner अपेक्षित आहे.[sih2026.vuce]

Exact problem

UGV ला खालील गोष्टी एकाच वेळी कराव्या लागतात:

- समोरचा terrain traversable आहे का हे ठरवणे

- rocks, ditch, trees, loose soil, water-like patches, humans, animals, vehicles यांसारखे hazards ओळखणे

- GPS नसताना स्वतःचे position आणि orientation estimate करणे

- local map तयार करून destination कडे route plan करणे

- अचानक अडथळा आल्यास सुरक्षित alternate route तयार करणे

- compute, battery, latency आणि limited sensor budget मध्ये हे real time करणे

Root cause

Outdoor autonomy चे खरे आव्हान object detection नसून terrain understanding under uncertainty आहे.

रस्त्यावर lane, curb, traffic rules आणि map असतात. पण unstructured outdoor terrain मध्ये:

- स्पष्ट boundaries नसतात

- माती, गवत, चिखल, gravel आणि shadow visually सारखे दिसू शकतात

- terrain दिसायला चालण्याजोगा असला तरी vehicle अडकू शकतो

- camera ला ditch चा depth, slope किंवा loose-soil traction स्पष्ट कळेलच असे नाही

- sun glare, shadow, dust, rain आणि vibration मुळे vision degrade होते

- GPS उपलब्ध नसल्यास localization drift वाढतो

Research मध्येही हेच दिसते: semantic segmentation फक्त “grass/road/rock” label देते; पण mobility साठी traction, bumpiness, deformability आणि vehicle-specific risk महत्त्वाचे असतात. TerraPN सारखी work robot-terrain interaction मधून terrain properties learn करण्यावर भर देते. WayFAST/WayFASTER नेही simple segmentation पेक्षा self-supervised traversability prediction आवश्यक असल्याचे दाखवले आहे.[arxiv][ieeexplore.ieee][arxiv]

कोण प्रभावित होतात?

- Search-and-rescue teams: ढिगारा, जंगल, पूरग्रस्त भाग, low-visibility zones

- Defence and border logistics: GPS-jammed किंवा communication-denied terrain

- Agriculture: शेतातील uneven soil, crop rows, irrigation channels

- Industrial inspection: mines, construction sites, solar farms, warehouses बाहेरील परिसर

- Disaster response: landslide, rubble, wildfire perimeter, flood-prone routes

- Delivery/support UGVs: campuses, large estates, remote facilities

आजची workflow पद्धत

आजचे साधारण उपाय:

- Manual remote control

- GPS waypoint navigation

- Lidar-heavy navigation stack

- Camera + object detector + basic obstacle avoidance

- Pre-mapped route आणि geofencing

- Human operator override

ही पद्धत limited परिस्थितीत चालते; पण GPS denied, changing terrain आणि low-cost vision-centric deployment मध्ये ती कमकुवत ठरते.

Existing solutions अपुरे का आहेत?

| सामान्य approach | तो काय करतो | का अपुरा आहे |
| --- | --- | --- |
| YOLO/object detection | Person, tree, rock, vehicle detect करतो | “हा terrain vehicle ला चालेल का?” हे सांगत नाही |
| Semantic segmentation | Grass, soil, road, water, rock labels देते | Same-class terrain ची traction किंवा sinkage risk वेगळी असू शकते |
| Basic visual SLAM | Camera features वरून pose estimate करतो | Repetitive grass, motion blur, low texture, dust, glare आणि rain मध्ये tracking lose होऊ शकते |
| GPS waypoint navigation | Global route आणि position देते | GPS jammed/spoofed/blocked झाल्यावर fail |
| Pure monocular camera | कमी खर्चात perception | Metric depth/scale ambiguous; ditch, slope आणि distance estimate unreliable होऊ शकते |
| Reactive obstacle avoidance | समोर obstacle आला की वळतो | पुढचा route traversable असेल याची खात्री नसते; cul-de-sac किंवा ditch मध्ये अडकू शकतो |
| Large LiDAR-centric stack | मजबूत 3D geometry | SIHच्या “vision-based” thrust, cost, power, payload आणि deployment constraints विरुद्ध जाऊ शकते |

Visual SLAM literature मध्ये uncertainty, feature matching, keyframe selection आणि map representation हे core open challenges आहेत; single-sensor uncertainty कमी करण्यासाठी multi-sensor fusion आवश्यक असल्याचे review स्पष्ट करते. Rain मध्ये visual odometry pipelines अपुरे ठरू शकतात आणि robust sensor fusionची गरज वाढते.[mdpi][arxiv][arxiv]

Problem setter ला नेमके काय अपेक्षित आहे?

Explicit expectations

- Lightweight perception AI

- Obstacle आणि path detection

- GPS-free visual localization

- Path planning

- Motor/wheel command generation

- Outdoor Point A-to-B autonomous navigation

- Collision-free operation[sih2026.vuce]

Hidden expectations

BEL/defence-oriented evaluation मध्ये judges बहुधा खालील गोष्टी अप्रत्यक्षपणे पाहतील:

- GPS नसताना system खरोखर चालतो का?

- Model demo-only आहे का, की real outdoor terrain वर reliable आहे?

- अचानक obstacle आला तर route recalculate होतो का?

- Safety fallback आहे का?

- Low-cost hardware वर latency acceptable आहे का?

- Network नसताना पूर्ण autonomy चालते का?

- Model decision explain करता येते का?

- System “I don’t know” म्हणू शकतो का?

- Fail झाल्यावर vehicle uncontrolled हालत नाही ना?

- Robustness: changing light, soil, grass, shadow, uneven terrain

Typical SIH team काय बनवेल?

बहुतेक teams अशी pipeline दाखवतील:

Camera → YOLO obstacle detection → OpenCV lane/path detection

→ ORB-SLAM → A* planner → Arduino motor control

ही demo साठी ठीक असली तरी तिच्या मोठ्या मर्यादा:

- Lane नसलेल्या terrain वर OpenCV path detection कमजोर

- YOLO “rock” शोधेल, पण loose soil किंवा slippery grass समजणार नाही

- ORB-SLAM drift किंवा tracking loss झाले तर system काय करेल?

- Planner vision confidence आणि terrain risk वापरणार नाही

- No confidence-aware stop policy

- No self-improvement from vehicle experience

- No vehicle-specific traversability learning

आमची differentiated solution: PRAGYA-Nav

मी solutionचे नाव सुचवतो:

PRAGYA-Nav: Predictive Risk-Aware Ground Autonomy for GPS-denied Outdoor Navigation

मराठीत सोपे नाव:

स्वयं-दृष्टी: अनुभवातून शिकणारी सुरक्षित UGV नेव्हिगेशन प्रणाली

One-line pitch

“PRAGYA-Nav is a camera-first, GPS-denied autonomous navigation stack that creates an uncertainty-aware terrain-risk map, learns traversability from the UGV’s own motion and vibration feedback, and chooses routes that are not merely shortest—but safest, driveable, and confidence-calibrated.”

Innovation काय आहे?

साध्या semantic class वर निर्णय घेण्याऐवजी प्रत्येक terrain cell साठी खालील तीन प्रश्न विचारले जातील:

- पुढे जाणे शक्य आहे का?

- पुढे जाणे किती सुरक्षित आहे?

- या निर्णयाबद्दल system ला किती confidence आहे?

यानंतर UGV shortest path निवडणार नाही. तो minimum-risk path निवडेल.

Key differentiator

Camera image ≠ only object labels

Camera image + visual geometry + IMU vibration + wheel slip + motor effort

↓

Self-supervised terrain experience label

↓

Terrain risk / traversability predictor

↓

Uncertainty-aware local semantic costmap

↓

Risk-aware planner + safe fallback behaviour

हा दृष्टिकोन WayFAST, WayFASTER आणि TerraPN सारख्या research direction शी जुळतो: terrain traversability manual labels वर अवलंबून न ठेवता robot experience द्वारे learn करता येते. WayFASTER RGB/depth sequence आणि pose data वापरून unseen/partially visible terrain बद्दल prediction सुधारते; TerraPN velocities, IMU vibration आणि odometry-error signals वापरून complex terrain properties शिकते.[arxiv][arxiv]

System architecture

Stereo/RGB camera + IMU + wheel encoders

│

▼

Sensor synchronisation layer

│

┌───────┴────────┐

▼                ▼

Visual-Inertial      Lightweight terrain

Odometry / SLAM      perception model

│                │

▼                ▼

Local pose +          Semantic classes +

map consistency       depth + uncertainty

│                │

└───────┬────────┘

▼

Self-supervised terrain assessor

┌──────────────────────────────┐

│ Slip estimate                │

│ Vibration/roughness          │

│ Wheel effort/current         │

│ Odometry disagreement        │

│ Terrain encounter history    │

└──────────────────────────────┘

│

▼

Risk-aware traversability costmap

│

▼

Local planner + dynamic obstacle avoidance

│

▼

Motor commands / ROS 2 control interface

│

▼

Safe-stop / return-to-known-safe-zone

Four-layer intelligence

1. Terrain perception layer

Input:

- Front RGB camera किंवा stereo/RGB-D camera

- IMU

- Wheel encoders

- Optional low-cost ToF/depth sensor for near-field safety

Output:

- Traversable terrain mask

- Hazard mask

- Obstacle location

- Estimated depth/slope proxy

- Dynamic-object mask

- Pixel-level uncertainty score

Suggested categories:

| Class | Navigation action |
| --- | --- |
| Compact soil / paved ground | Low cost, preferred |
| Short grass | Medium cost, permitted |
| Tall grass | Medium-high cost; slow and verify |
| Gravel | Medium-high cost; reduce speed |
| Mud / loose sand | High risk; avoid unless verified |
| Water / reflective surface | Lethal cost; avoid |
| Ditch / trench | Lethal cost; avoid |
| Rock / tree / wall | Lethal cost; avoid |
| Human / animal | Dynamic safety zone and stop/slow policy |
| Unknown terrain | High uncertainty; explore only at crawl speed |

2. Visual localization layer

Recommended primary method:

- ORB-SLAM3 or VINS-Fusion style visual-inertial odometry

- Camera + IMU fusion

- Wheel odometry as a scale and motion-consistency constraint

- Loop closure when returning to known places

- Local submaps instead of requiring perfect global map

Why visual-inertial?

Monocular-only SLAM मध्ये scale ambiguity, blur आणि texture-less patches मुळे failureचा धोका असतो. Visual + IMU + wheel odometry fusion हा stronger, practical आणि SIH-suitable design आहे. GPS-denied field settings मध्ये visual-inertial and additional odometry fusionची गरज research literature मध्येही अधोरेखित केली आहे.[mdpi][mdpi][mdpi]

3. Self-supervised terrain learning layer

हे तुमचे सर्वात मजबूत innovation आहे.

Terrain Experience Vector

UGV जेव्हा एखाद्या terrain patch वरून जाईल, तेव्हा system खालील signals record करेल:

E_t =

[w_{\text{slip}},

a_{\text{rms}},

\Delta v,

e_{\text{odom}},

i_{\text{motor}},

\theta_{\text{roll}},

\theta_{\text{pitch}}]

जिथे:

- w_{\text{slip}}: wheel slip estimate

- a_{\text{rms}}: IMU vibration magnitude

- \Delta v: commanded आणि actual velocity मधील फरक

- e_{\text{odom}}: vision आणि wheel odometry disagreement

- i_{\text{motor}}: motor current/effort

- \theta_{\text{roll}}: roll angle

- \theta_{\text{pitch}}: pitch/slope behaviour

Traversability score

T = 1 - \sigma

\left(

\alpha w_{\text{slip}}+

\beta a_{\text{rms}}+

\gamma e_{\text{odom}}+

\delta i_{\text{motor}}+

\epsilon |\theta_{\text{roll}}|

\right)

जिथे:

- T \in [0,1]

- T \approx 1: safe and easy to drive

- T \approx 0: unsafe/non-traversable

- \sigma: normalizing sigmoid function

यानंतर image patch आणि terrain-experience score pair करून online learning किंवा periodic incremental learning करणे.

Result

UGV सुरुवातीला grass ला medium risk मानू शकतो. पण जर त्या grass वर slip कमी, vibration कमी आणि velocity stable असेल तर तो terrain पुढच्या वेळी safe म्हणून ओळखेल. उलट दिसायला साधी माती असून motor current वाढतो आणि wheel slip होतो, तर system ती high-risk terrain म्हणून map करेल.

यालाच Experience-Calibrated Traversability म्हणा.

4. Risk-aware planner

Normal A* path planner:

J_{\text{normal}} =

\text{Distance}

PRAGYA-Nav planner:

J_{\text{PRAGYA}} =

\alpha D +

\beta R_{\text{terrain}} +

\gamma U_{\text{perception}} +

\delta S_{\text{slope}} +

\eta O_{\text{obstacle}} +

\zeta E_{\text{energy}}

जिथे:

- D: route distance

- R_{\text{terrain}}: terrain traversability risk

- U_{\text{perception}}: uncertainty

- S_{\text{slope}}: slope/roughness cost

- O_{\text{obstacle}}: static and dynamic obstacle risk

- E_{\text{energy}}: battery/effort cost

यामुळे vehicle 5 m कमी असलेला पण चिखलाचा route टाळून 8 mचा सुरक्षित route निवडेल.

Safety-first behaviour

Safety ही तुमची judge-winning feature ठरू शकते.

Risk state machine

| State | Condition | UGV action |
| --- | --- | --- |
| Green | High confidence, low terrain risk | Normal autonomous speed |
| Yellow | Moderate uncertainty/risk | Speed reduce, increase observation |
| Orange | High uncertainty, ambiguous terrain | Crawl mode, replan, seek safer route |
| Red | Collision / ditch / human / SLAM lost | Controlled stop |
| Recovery | Localization degraded | Return to last verified safe waypoint or request operator intervention |

“Do not guess” rule

जर terrain prediction confidence कमी असेल:

Do not accelerate.

Do not enter unknown patch blindly.

Slow down → observe more frames → re-estimate → replan.

If confidence stays low → stop.

हा feature साध्या SIH projectsपेक्षा system ला professional robotics flavour देईल.

Computer vision strategy

Model recommendation

Practical approach:

- Base model: YOLOv8n-seg / YOLO11n-seg equivalent lightweight segmentation

- Terrain segmentation: MobileNetV3-DeepLabV3+ / BiSeNetV2 / Fast-SCNN

- Depth: Stereo disparity किंवा lightweight monocular depth model

- Uncertainty: Monte Carlo dropout, ensemble-lite किंवा segmentation confidence entropy

- Optimization: ONNX Runtime / TensorRT / OpenVINO

का single YOLO पुरेसा नाही?

YOLO object boxes देतो. पण UGVला pixel-level “चलण्याजोगा ground” आणि local terrain cost हवा असतो. म्हणून architecture मध्ये दोन parallel branches:

Branch A: Object detection

Human, tree, rock, vehicle, animal, cone, obstacle

Branch B: Semantic traversability

Soil, grass, gravel, mud, water, shadow, ditch, unknown

Fusion:

Depth + object mask + traversability score + uncertainty

Dataset strategy

Public datasets वापरा, पण final model स्वतःच्या field recordings ने fine-tune करा.

- RUGD: Rough Unstructured Ground Driving Dataset

- RELLIS-3D: off-road segmentation and terrain data

- Cityscapes/BDD100K: generic objects आणि lighting robustness

- KITTI/Oxford RobotCar: visual odometry baseline validation

- स्वतःचा “SIH-Outdoor-Terrain” micro-dataset

स्वतःचा dataset कसा special करायचा?

कमीत कमी खालील conditions capture करा:

- Morning / noon / evening

- Direct sunlight आणि deep shadow

- Dry soil / wet soil / gravel / grass

- Small rocks / tree roots / shallow ditch

- Static obstacle आणि suddenly introduced obstacle

- Camera vibration

- Narrow path

- Unknown terrain

- Human crossing

Each frame सोबत:

RGB image

Timestamp

IMU

Wheel encoder

Motor PWM/current

Vehicle velocity

Slip proxy

Traversability label

Obstacle label

System confidence

तुमचा dataset स्वतःच solutionचा research contribution बनेल.

Hardware suggestion

Problem software category मध्ये असला तरी prototype साठी real vehicle demonstration मोठा advantage देईल.

| Component | Budget-friendly choice | Role |
| --- | --- | --- |
| Compute | NVIDIA Jetson Orin Nano / Raspberry Pi 5 + Coral TPU | Onboard AI inference |
| Camera | Intel RealSense D435i / OAK-D Lite / stereo USB camera | RGB, depth, IMU where available |
| IMU | BNO085 / ICM-20948 | Orientation, vibration, pitch/roll |
| Wheel encoder | Quadrature encoder | Motion and slip estimation |
| Motor controller | BTS7960 / Cytron / Roboclaw | Closed-loop motor control |
| Chassis | Differential-drive UGV | Rapid prototype |
| Emergency stop | Physical switch + software e-stop | Mandatory safety |
| Optional sensing | Short-range ToF/ultrasonic | Last-line collision safety |

Important positioning

तुमचे pitch असे असावे:

“Camera is the primary semantic sensor. IMU and wheel encoders are not replacing vision; they validate vision through proprioception and make GPS-denied autonomy safer.”

यामुळे तुम्ही problem statementशी aligned राहता आणि practical robustnessही मिळते.

Software stack

Ubuntu 22.04

ROS 2 Humble / Jazzy

Python + C++ nodes

OpenCV

PyTorch for training

ONNX Runtime / TensorRT for deployment

ORB-SLAM3 / VINS-Fusion

Nav2

RTAB-Map optionally for map visualization

Foxglove / RViz2 dashboard

SQLite/ROS bag for terrain-experience memory

ROS 2 node graph

/camera_node

/imu_node

/wheel_encoder_node

/visual_inertial_odometry_node

/terrain_segmentation_node

/object_detection_node

/depth_estimation_node

/uncertainty_estimator_node

/terrain_experience_node

/risk_costmap_node

/nav2_planner_node

/local_controller_node

/safety_supervisor_node

/motor_controller_node

/dashboard_node

ROS 2 Nav2 मध्ये semantic segmentation आणि terrain mapping output ला costmap layer म्हणून integrate करण्याचे open-source patterns उपलब्ध आहेत.[github][github]

Technical feasibility

MVP मध्ये काय बनवा?

पहिल्या working prototype मध्ये:

- Outdoor video feed

- Traversable/non-traversable segmentation

- Rock/tree/person detection

- Visual-inertial odometry

- Local occupancy + risk costmap

- A* / D* Lite global planning

- Dynamic Window Approach किंवा MPPI local planner

- UGV motor command output

- Safe stop on obstacle/high uncertainty

- Live dashboard showing pose, terrain risk, confidence and selected route

Advanced feature

Final demo मध्ये दाखवा:

- UGV एका grass/soil patchवरून पहिल्यांदा जाते

- Wheel slip आणि vibration observe करते

- त्या patchला “risky” म्हणून terrain memory मध्ये store करते

- दुसऱ्या run मध्ये ती patch avoid करते किंवा slow mode घेते

- अचानक obstacle ठेवला जातो

- UGV reroute करते

- Camera blind/shadow scenario मध्ये confidence कमी झाल्यावर controlled stop करते

हा “learn, remember, adapt” demonstration judgesना खूप प्रभावी वाटेल.

Evaluation framework

फक्त “robot चालला” असे म्हणू नका. Quantitative evaluation करा.

| Metric | Target |
| --- | --- |
| Path completion rate | 85%+ controlled outdoor trials |
| Collision rate | 0 collision in test set |
| Obstacle detection recall | 90%+ for test obstacles |
| Traversability classification F1 | 80%+ initially; improve after calibration |
| Localization drift | Less than 2–5% of travelled distance in test course |
| Replanning latency | Less than 500 ms for local obstruction |
| End-to-end perception latency | Less than 150 ms on target hardware |
| Safe-stop response | Less than 300 ms for critical obstacle class |
| Terrain adaptation gain | Lower slip / fewer interventions after learning |
| Energy efficiency | Compare safe route vs shortest route |

Ablation study

Judgesना हे दाखवा:

| Version | Expected limitation |
| --- | --- |
| Object detection only | Sees objects but misunderstands terrain |
| Segmentation + planner | Understands labels but not vehicle-specific risk |
| SLAM + segmentation | Navigates but may take risky terrain |
| PRAGYA-Nav full system | Learns terrain, quantifies uncertainty, replans safely |

हा comparison तुमच्या innovation ला academic आणि engineering दोन्ही credibility देईल.

Research-backed gaps

Gap 1: Semantics विरुद्ध traversability

“Grass” label म्हणजे UGV तेथे safely जाऊ शकतेच असे नाही. Terrain geometry, traction आणि vehicle dynamics महत्त्वाचे आहेत. Traversability research मध्ये semantic segmentation सोबत terrain-aware modellingची गरज अधोरेखित केली जाते.[link.springer][ieeexplore.ieee]

Gap 2: Offline datasetsवर over-reliance

Traditional perception models static training data वर आधारित असतात. पण field terrain बदलते. Self-supervised learning terrain contact signal वापरून task environmentशी adapt होऊ शकते.[arxiv][ieeexplore.ieee]

Gap 3: Camera confidence दुर्लक्षित

Models अनेकदा wrong prediction देतात, पण planner त्यावर पूर्ण विश्वास ठेवतो. तुमच्या solutionमध्ये uncertainty स्वतंत्र cost म्हणून वापरली जाईल.

Gap 4: “Shortest path” bias

A* आणि common planners distance reduce करतात; safety आणि mobility costसाठी route optimize करणे आवश्यक आहे.

Gap 5: Localization failures साठी safe recovery नसणे

SLAM tracking lost झाल्यावर normal systemsचे behaviour अस्पष्ट असते. PRAGYA-Nav मध्ये stop, local relocalization, and safe-return policy असेल.

Novelty claim

तुमच्या PPT मध्ये हा novelty statement वापरू शकता:

“Unlike conventional vision-only obstacle avoidance systems, PRAGYA-Nav closes the perception-action-learning loop. It converts real vehicle behaviour—wheel slip, vibration, odometry inconsistency, and motor effort—into self-supervised terrain labels. These labels continuously calibrate a visual traversability model, enabling safe route planning under GPS denial and environmental uncertainty.”

Patentable angle

Potential IP direction:

“A method for uncertainty-aware, self-supervised terrain traversability estimation for GPS-denied unmanned ground vehicles using fused visual perception, wheel-slip proxy, inertial vibration, odometry disagreement, and motor-effort feedback.”

Patents शोधताना तुम्ही “terrain traversability estimation,” “self-supervised mobile robot navigation,” “vision-based UGV terrain classification,” आणि “risk-aware autonomous path planning” या keywordsने prior-art search करा. तुमचा novelty claim “self-supervised feedback loop + uncertainty gated planning + safe behavioural policy” यावर केंद्रित ठेवा.

Demo scenario

Demo setup

एक outdoor test track तयार करा:

- Soil path

- Short grass patch

- Gravel patch

- Rock obstacle

- Small tree/box obstacle

- Shallow ditch किंवा black mat representing non-traversable zone

- Wet/reflective patch simulation

- Sudden obstacle placed during execution

- GPS disabled/mobile hotspot off

Demo narrative

- Operator destination निवडतो

- UGV GPS शिवाय visual-inertial localization सुरू करते

- Terrain segmentation आणि depth-based hazard map तयार होते

- Risk-aware route निवडला जातो

- UGV grass patchमध्ये slip detect करते

- System terrain risk update करते

- पुढे box अचानक ठेवला जातो

- UGV stop/slow करते, map update करते आणि reroute करते

- Dark-shadow/uncertain regionमध्ये confidence कमी होते

- UGV crawl mode किंवा safe stop चालू करते

- Dashboardवर “why” दिसते: “High uncertainty due to glare; speed limited to 0.15 m/s”

हे सामान्य SIH demo पेक्षा खूप advanced दिसेल.

30-day execution plan

Days 1–5: Base platform

- ROS 2 setup

- Camera, IMU, encoder integration

- Differential drive motor control

- Manual teleoperation

- Recording pipeline तयार करा

- Outdoor dataset capture सुरू करा

Days 6–10: Perception

- Terrain segmentation baseline

- Object detection baseline

- RGB/depth alignment

- Basic class-to-cost mapping

- On-device inference optimization

Days 11–15: Localization

- ORB-SLAM3/VINS-Fusion integration

- IMU calibration

- Wheel encoder fusion

- Pose visualization in RViz2

- Map storage and replay

Days 16–20: Navigation

- Occupancy/risk costmap

- A* / D* Lite planner

- Dynamic obstacle layer

- Local controller

- Emergency stop policy

Days 21–25: Differentiator

- Slip proxy calculation

- IMU vibration feature extraction

- Motor effort feature

- Terrain experience memory

- Risk score update

- Confidence-aware speed controller

Days 26–30: Testing and pitch

- Test in 3–5 terrain conditions

- Baseline comparison

- Record failures honestly

- Tune safety thresholds

- Prepare dashboard/video/PPT

- Practice judges’ questions

SIH PPT structure

- Problem statement and mission context

- Why conventional camera navigation fails outdoors

- Research gap: semantics is not traversability

- Proposed system: PRAGYA-Nav

- Architecture diagram

- Terrain Experience Vector and self-supervised learning

- Uncertainty-aware risk costmap

- GPS-denied visual-inertial localization

- Safety state machine

- Prototype hardware/software stack

- Dataset and field-testing methodology

- Quantitative metrics and baseline comparison

- Demo workflow

- Novelty, scalability and BEL relevance

- Future roadmap: multi-UGV, thermal/radar fallback, federated terrain learning

Judges काय विचारू शकतात?

“Camera-only असल्यावर depth कशी मिळवणार?”

उत्तर:

“Camera remains the primary perception modality. For robust near-field geometry, we use stereo/depth estimation; IMU and wheel encoders provide proprioceptive validation, not a replacement for visual understanding. This is important because pure monocular vision has scale ambiguity and can degrade under glare, rain, and low-texture terrain.”[arxiv][mdpi]

“YOLO वापरून हे करता येणार नाही का?”

उत्तर:

“YOLO detects discrete objects. Outdoor navigation needs a dense, vehicle-specific traversability field. A grass patch may be safe for one UGV and unsafe for another due to wheel type, payload, soil moisture, and slope. We therefore combine semantic perception with measured vehicle-terrain interaction.”

“GPS नाही तर global target कसा define कराल?”

उत्तर:

“Mission startवर local coordinate frame initialize करू. Destination can be selected visually on an operator-provided local map, as a relative coordinate, through a fiducial marker, or via visual place-goal identification. The vehicle performs local visual-inertial navigation and uses loop closure for map consistency.”

“SLAM हरवले तर काय?”

उत्तर:

“Safety supervisor marks localization confidence low. The UGV immediately slows/stops, attempts relocalization against recent keyframes, and if recovery fails, returns only through last known safe trajectory or requests operator intervention.”

“तुमची खरी innovation काय?”

उत्तर:

“Closed-loop self-supervised traversability learning. The UGV does not merely classify what it sees; it learns whether the terrain was actually drivable by comparing visual prediction with slip, vibration, motor effort, and odometry consistency, then uses that learned risk in future route planning.”

Final recommendation

तुमचा final solution एक साधा autonomous robot म्हणून pitch करू नका. त्याला “risk-aware field autonomy platform for GPS-denied operations” म्हणून position करा.

तुमचे winning differentiators:

- Vision-first but sensor-validated architecture

- GPS-denied visual-inertial localization

- Terrain-specific, vehicle-specific traversability assessment

- Self-supervised learning from actual UGV-terrain interaction

- Uncertainty-aware planning

- Dynamic rerouting

- Safe-stop and recovery behaviour

- Edge-deployable lightweight stack

- Field-test metrics rather than only simulation claims

- Explainable dashboard: “robot avoided this terrain because risk/confidence/slip estimate was high”

BELच्या stated need—outdoor, GPS-denied, camera-primary safe autonomy—शी हे पूर्णपणे aligned आहे, पण routine object detection demo पेक्षा लक्षणीय अधिक mature आणि differentiated आहे.[sih2026.vuce]